MODEL 01 : TRANSMISSION SPECTRA CALCUALTION TROUGH MUMAX3

MuMax3 does not calculate the transmission analytically; instead, it solves the time-dependent Landau–Lifshitz–Gilbert (LLG) equation, which is described by Eq. (1) below:

dmdt= -γm × Heff+ αm × dmdt (1)

Where Heffincludes the exchange field, demagnetizing field, anisotropy, and external excitation fields. The magnetization M(r,t)is obtained in real space and time.
The transmission spectrum is obtained in MuMax3 through the sequence of steps listed below.

1. Spin-Wave Excitation

- Spin waves are excited using a broadband excitation, or

- A continuous wave (CW) at a single frequency.

An example of a well-known broadband excitation that can be inserted into a MuMax3 simulation file is:

- B_ext.add(mask, A*sinc(2*pi*fmax*(t-t0)))

This injects a known spin-wave power into the magnon propagation waveguide.

2. Definition of Probe Regions

Two specific probe regions are defined:

- Input region: spin-wave propagation before the ring resonator

- Output region: spin-wave propagation after interaction with the resonator

For each probe, the z-component of the magnetization vector and its squared magnitude are recorded, as illustrated below:

- mz_in := Crop(mz, x1, x2, ...)

- mz_out := Crop(mz, x3, x4, ...)

3. Fourier Transform (Frequency Domain)

This is arguably the most important step, as it allows one to extract a large amount of information using signal-processing techniques.
The Fast Fourier Transform (FFT) applied to the time-domain magnetization signals mz(t)determines the input and output intensity spectra, Iin(f)and Iout(f), which are proportional to the squared magnitude of the Fourier-transformed magnetization vectors.

4. Transmission Spectra

The calculation of the transmission spectrum is obtained by evaluating the ratio of the spin-wave intensities in the straight waveguide measured before and after the passage through the ring resonator:

Tf= Iout(f)Iin(f)= |mzoutf|2|mzinf|2 (2)